// Handyboard intrinsics

// RESERVED WORDS

// KEYWORD #define

<pre>#define macro expression</pre><br>

The #define command creates textual replacements in C.  For example,

if you were to have the following:<br><br>

<pre>#define PI 3.14159</pre>

then any time PI occurs in your program, it will be replaced by

3.14159.  #define does not affect text within quotes, and only

replaces whole words, so the following would not be affected by the

above define:<br><br>

<pre>SPIED

"I love PI"</pre>

A #define statement only affects the remainder of the file after the

#define.<br><br>

Your #define macro can also take arguments.  Here's an example:<br><br>

<pre>#define fdslow(x) motor(x,50)</pre>

This example would replace fdslow(1) with motor(1,50)

// KEYWORD #else

// KEYWORD #endif

// KEYWORD #error

// KEYWORD #if

// KEYWORD #ifdef

// KEYWORD break

<pre>break;<pre>

The break statement causes the enclosing for or while loop to stop

immediately, and *for execution to resume at the next statement

following the loop.<br><br>

The break statement is only valid inside a *for* or *while* loop.<br><br>

A simple example, which waits until the start button is pressed, then

beeps:<br><br>

<pre>

while (1) {

if (start_button()) {

// Start button got pressed

// Stop the loop, go straight to first statement after loop

break;

}

}

beep(); // first statement after loop

</pre>

If break is used within nested loops, it only causes the innermost

loop to exit.  This example program tests four motors sequentially,

until the stop button is pressed:

<pre>

int m;

// Test each motor, one at a time, until the stop button is pressed

// When done, beep

while (1) {

for (m= 0; m <= 3; m++) {

fd(m);

sleep(0.5);

if (stop_button()) break;

bk(m);

sleep(0.5);

if (stop_button()) break;

off(m)

}

// The break statements inside the for loop only exit that loop,

// We need to check again and call break from here to leave the while loop

if (stop_button()) break;

}

alloff();

beep();   // done!

</pre>

See also: for, while, continue

// KEYWORD continue

<pre>

continue;

</pre>

The continue statement causes the enclosing for or while loop to skip

the rest of the loop body for the current iteration.  Unlike break,

continue does not stop the loop.  Instead, if the loop still wants to

execute (the while or for condition is true), the loop will continue

with the next iteration.<br><br>

Example:<br>

<pre>

// Test each motor, but skip motor 2

int m;

for (m= 0; m <= 3; m++) {

if (m == 2) continue; // skip to next iteration

fd(m);

sleep(0.5);

bk(m);

sleep(0.5);

off(m);

}

// KEYWORD char

<pre>

char foo[4];

char foo[4]="abc";

</pre>

The char type represents a number from 0 to 255, and can be used to

represent a single text character using ASCII encoding.  In

Interactive C, the char type is only used in arrays, typically for

strings.  If you want to express a single number, use an int or a long

instead.

// LINK ASCII

Ascii characters, and their integer values.

Ascii values 0 to 31, and 127 are not printable.

<pre>

Dec   Char           Dec   Char

------------------------------------

0     NUL            64    @

1     SOH            65    A

2     STX            66    B

3     ETX            67    C

4     EOT            68    D

5     ENQ            69    E

6     ACK            70    F

7     BEL            71    G

8     Backspace      72    H

9     HT             73    I

10    LF             74    J

11    VT             75    K

12    FF             76    L

13    CR             77    M

14    SO             78    N

15    SI             79    O

16    DLE            80    P

17    DC1            81    Q

18    DC2            82    R

19    DC3            83    S

20    DC4            84    T

21    NAK            85    U

22    SYN            86    V

23    ETB            87    W

24    CAN            88    X

25    EM             89    Y

26    SUB            90    Z

27    ESC            91    [

28    FS             92    \

29    GS             93    ]

30    RS             94    ^

31    US             95    _

32    Space          96    `

33    !              97    a

34    "              98    b

35    #              99    c

36    $              100   d

37    %              101   e

38    &              102   f

39    '              103   g

40    (              104   h

41    )              105   i

42    *              106   j

43    +              107   k

44    ,              108   l

45    -              109   m

46    .              110   n

47    /              111   o

48    0              112   p

49    1              113   q

50    2              114   r

51    3              115   s

52    4              116   t

53    5              117   u

54    6              118   v

55    7              119   w

56    8              120   x

57    9              121   y

58    :              122   z

59    ;              123   {

60    <              124   |

61    =              125   }

62    >              126   ~

63    ?              127   Delete

</pre>

// KEYWORD float

// KEYWORD for

// for (initial; condition; aftereach) { body; }

// For loop.

// KEYWORD if

// if (test) { body1; } else { body2; }

// If test is true (non-zero), execute body1.

// Otherwise execute body2 (optional)

<pre>

if (<i>test</i>) { <i>body;</i> }

</pre>

The simplest if statement has two parts: a test and a body.  First,

the test is performed.  Second, *if the test succeeded, body will be

executed.<br><br>

Example:<br>

<pre>

// If analog sensor 4 reads higher than 100, turn motor 0 on forwards

if (analog(4)>100) {

fd(0);

}

</pre>

The if statement can also have an <b>else</b> clause.

if

1. Test the condition

// KEYWORD else

// KEYWORD int

// KEYWORD long

// KEYWORD persistent

// KEYWORD return

// KEYWORD struct

// KEYWORD typedef

// KEYWORD union

// KEYWORD void

// KEYWORD while

// while (condition) { body; }

// Loop while condition is true (non-zero)

// Functions (builtin and otherwise)

// DOC alloff

// alloff()

// Turn motors 0-3 off

// DOC analog

// int analog(int: channel)

// Read analog sensor, returns 0-255

// DOC ao

// ao()

// Turn motors 0-3 off

// DOC atan

// float atan(float)

// Arctangent (in radians)

// DOC beep

// beep()

// Make a quick beep

// DOC beeper_off

// void beeper_off()

// Turn the beeper off (use with beeper_on and set_beeper_pitch)

// DOC beeper_on

// void beeper_on()

// Turn the beeper on (call set_beeper_pitch first)

// DOC bk

// void bk(int: motor 0-3)

// Turn motor on in "backwards" directionint)

// DOC clear_digital_out

// clear_digital_out(int: channel)

// Set expansion board digital output to logic low

// DOC cos

// float cos(float)

// Cosine of angle (radians)

// DOC defer

// defer()

// Switch to another process now

// DOC digital

// int digital(int)

// Read digital sensor.  1 for button pressed, 0 for not pressed

// DOC disable_encoder

// void disable_encoder(int: channel 0-3)

// Disable encoder channel

// DOC enable_encoder

// void enable_encoder(int: channel 0-3)

// Enable encoder channel

// DOC exp10

// float exp10(float: x)

// Return 10 to the power of x

// DOC exp

// float exp(float: x)

// Return e to the power of x

// DOC fd

// void fd(int: motor 0-3)

// Turn motor on in "forwards" directionint)

// DOC hog_processor

// void hog_processor()

// Keep from switching to another process

// DOC init_expbd_servos

// init_expbd_servos(int action)

// Enable expansion board servos if action=1.

// Disable expansion board servos if action=0.

// DOC kill_process

// void kill_process(int: pid)

// Kill process # pid (pid is returned by start_process)

// DOC knob

// int knob()

// Current position of knob, 0 to 255

// DOC log10

// float log10(float: x)

// Returns the log (base 10) of x

// DOC log

// float log(float)

// Returns the log (base e) of x

// DOC motor

// motor(int: motor 0 to 3, int: speed -100 to 100)

// Turn motor on at given speed

// DOC mseconds

// long mseconds()

// Return the number of milliseconds since the board was reset

// DOC msleep

// msleep(long: duration in milliseconds)

// Wait for given duration

// DOC off

// void off(int: motor 0-3)

// Turn motor off

// DOC printf

// void printf(string: "format", args...)

// Print to the LCD.  Examples:

// printf("Hello\n");

// printf("analog(3) is %d\n", analog(3));

// DOC random

// int random(int)

// Return a random number between 0 and num-1, inclusive

// DOC read_encoder

// int read_encoder(int: channel 0-3)

// Gives the number of counts for encoder (must call enable_encoder)

// DOC reset_encoder

// reset_encoder(int: channel 0-3)

// Reset count for encoder

// DOC reset_system_time

// void reset_system_time()

// Reset the system time to 0

// DOC seconds

// float seconds()

// Return the number of seconds since the board was reset

// DOC servo0

// int servo0

// Set this variable to control servo 0 (must call init_expbd_servos first)

// DOC servo1

// int servo1

// Set this variable to control servo 1 (must call init_expbd_servos first)

// DOC servo2

// int servo2

// Set this variable to control servo 2 (must call init_expbd_servos first)

// DOC servo3

// int servo3

// Set this variable to control servo 3 (must call init_expbd_servos first)

// DOC servo4

// int servo4

// Set this variable to control servo 4 (must call init_expbd_servos first)

// DOC servo5

// int servo5

// Set this variable to control servo 5 (must call init_expbd_servos first)

// DOC set_beeper_pitch

// void set_beeper_pitch(float: frequency)

// Set the beeper pitch (use with beeper_on and beeper_off)

// DOC set_digital_out

// int set_digital_out(int)

// Set expansion board digital output to logic high

// DOC sin

// float sin(float)

// Sine of angle (radians)

// DOC sleep

// void sleep(float: duration in seconds)

// Wait for given duration

// DOC sonar

// int sonar()

// Fires sonar and reports approximate distance in

// millimeters.  32767 means no echo detected.

// DOC sqrt

// float sqrt(float)

// Square root

// DOC start_button

// int start_button()

// Return 1 if start button pressed, 0 if not

// DOC start_press

// start_press()

// Wait for start button to be pressed

// DOC start_process

// int start_process(function call)

// Start a new process

// DOC stop_button

// int stop_button()

// Return 1 if stop button pressed, 0 if not

// DOC stop_press

// stop_press()

// Wait for stop button to be pressed

// DOC tan

// float tan(float: angle)

// Tangent of angle (radians)

// DOC test_digital_out

// int test_digital_out(int: channel)

// Return the last commanded state of the expansion board digital output

// DOC tone

// tone(float: frequency, float: duration in seconds)

// Make a tone

# Not worth templatizing

lib   beeper_on

lib   beeper_off

lib   set_beeper_pitch

lib   fd

doc   fd fd(int: motor 0-3)      Turn motor on in "forwards" direction

lib   bk

doc   bk bk(int: motor 0-3)      Turn motor on in "forwards" direction

lib   off

doc   off off(int: motor 0-3)      Turn motor off

lib   brake

doc   brake brake(...)    THIS FUNCTION DOES NOT EXIST FOR THE HANDYBOARD

lib   alloff

doc   alloff alloff()      Turn all motors off

lib   ao

doc   ao ao()      Turn all motors off

lib   motor

doc   motor motor(int: motor 0 to 3, int: speed -100 to 100)      Turn motor on at given speed

lib   stop_button

doc   stop_button stop_button()      Return 1 if stop_button is pressed

lib   start_button

doc   start_button start_button()      Return 1 if start_button is pressed

lib   knob

doc   knob knob()      Returns current position of knob, 0 to 255

lib   hog_processor

doc   hog_processor hog_processor()     Keep from switching to another process

lib   defer

doc   defer defer()      Switch to another process now

lib	start_process

doc 	start_process(function)      Start a new process.

lib	defer

doc	kill_process(int: pid)      Kill process # pid (pid is returned by start_process)

lib   random

doc   random(int: num)      Return a random number between 0 and num-1, inclusive

lib   printf

doc   printf printf(string: "format", args...)      Print to the LCD.  \

Examples: printf("Hello\n");  \

printf("analog(3) is %d\n", analog(3));

lib   servo0

doc   servo0 int servo0:  Set this variable to control servo 0. \

(Must call init_expbd_servos first)

lib   servo1

doc   servo1 int servo1:  Set this variable to control servo 1. \

(Must call init_expbd_servos first)

lib   servo2

doc   servo2 int servo2:  Set this variable to control servo 2. \

(Must call init_expbd_servos first)

lib   servo3

doc   servo3 int servo3:  Set this variable to control servo 3. \

(Must call init_expbd_servos first)

lib   servo4

doc   servo4 int servo4:  Set this variable to control servo 4. \

(Must call init_expbd_servos first)

lib   servo5

doc   servo5 int servo5:  Set this variable to control servo 5. \

(Must call init_expbd_servos first)

lib   init_expbd_servos

doc   init_expbd_servos init_expbd_servos(int action)      Enable expansion \

board servos if \

action=1.  Disable expansion board \

servos if action=0.

lib   sonar

doc   sonar int sonar()      Fires sonar and reports approximate distance in \

millimeters.  32767 means no echo detected.

#

# Standard C syntax constructs

#

//