/////////////////////

//  RESERVED WORDS //

/////////////////////

// KEYWORD #define

// KEYWORD #else

// KEYWORD #endif

// KEYWORD #error

// KEYWORD #if

// KEYWORD #if

// KEYWORD #ifdef

// KEYWORD break

// KEYWORD char

// KEYWORD continue

// KEYWORD else

// KEYWORD float

// KEYWORD for

// for (initial; condition; aftereach) { body; }

// For loop.

// KEYWORD if

// if (condition) { body1; } else { body2; }

// If condition is true (non-zero), execute body1.

// Otherwise execute body2 (optional)

// KEYWORD int

// KEYWORD long

// KEYWORD persistent

// KEYWORD return

// KEYWORD struct

// KEYWORD typedef

// KEYWORD union

// KEYWORD void

// KEYWORD while

// while (condition) { body; }

// Loop while condition is true (non-zero)

///////////////////////////////////////

// Functions (builtin and otherwise) //

///////////////////////////////////////

// DOC alloff

// alloff()

// Turns motors 0-3 off

// DOC analog

// int analog(int: channel)

// Reads analog sensor, returns 0-255

// DOC ao

// ao()

// Turns motors 0-3 off

// DOC atan

// float atan(float)

// Arctangent (in radians)

// DOC beep

// beep()

// Makes a quick beep

// DOC beeper_off

// beeper_off()

// Turns the beeper off (use with beeper_on and set_beeper_pitch)

// DOC beeper_on

// beeper_on()

// Turns the beeper on (call set_beeper_pitch first)

// DOC bk

// bk(int: motor 0-3)

// Turns motor on in "backwards" direction

// DOC clamp_camera_yuv

// clamp_camera_yuv ()

// Performs white balance script for CMUcam

// DOC clear_digital_out

// clear_digital_out(int: channel)

// Sets expansion board digital output to logic low

// DOC cos

// float cos(float: angle)

// Cosine of angle (radians)

// DOC defer

// defer()

// Switches to another process now

// DOC digital

// int digital(int)

// Reads digital sensor.  1 for button pressed, 0 for not pressed

// DOC disable_encoder

// disable_encoder(int: channel 0-3)

// Disables encoder channel

// DOC enable_encoder

// enable_encoder(int: channel 0-3)

// Enables encoder channel

// DOC exp10

// float exp10(float: x)

// Returns 10 to the power of x

// DOC exp

// float exp(float: x)

// Returns e to the power of x

// DOC fd

// fd(int: motor 0-3)

// Turns motor on in "forwards" direction

// DOC hog_processor

// hog_processor()

// Keeps from switching to another process

// DOC init_camera

// init_camera()

// Enables CMUcam in YUV mode and checks comm

// DOC init_expbd_servos

// init_expbd_servos(int action)

// Enables expansion board servos if action is 1,

// disables if 0

// DOC kill_process

// kill_process(int: pid)

// Kills process # pid (pid is returned by start_process)

// DOC knob

// int knob()

// Current position of knob, 0 to 255

// DOC log10

// float log10(float: x)

// Returns the log (base 10) of x

// DOC log

// float log(float)

// Returns the log (base e) of x

// DOC motor

// motor(int: motor 0 to 3, int: speed -100 to 100)

// Turns motor on at given speed

// DOC mseconds

// long mseconds()

// Returns the number of milliseconds since the board was reset

// DOC msleep

// msleep(long: duration in milliseconds)

// Waits for given duration

// DOC off

// off(int: motor 0-3)

// Turns motor off

// DOC printf

// printf(string: "format", args...)

// Prints to the LCD.  Examples:

// printf("Hello\n");

// printf("analog(3) is %d\n", analog(3));

// DOC random

// int random(int)

// Returns a random number between 0 and num-1, inclusive

// DOC read_encoder

// int read_encoder(int: channel 0-3)

// Gives the number of counts for encoder (must call enable_encoder)

// DOC reset_encoder

// reset_encoder(int: channel 0-3)

// Resets count for encoder

// DOC reset_system_time

// reset_system_time()

// Resets the system time to 0

// DOC seconds

// float seconds()

// Returns the number of seconds since the board was reset

// DOC servo0

// int servo0

// Set this variable to control servo 0 (must call init_expbd_servos first)

// DOC servo1

// int servo1

// Set this variable to control servo 1 (must call init_expbd_servos first)

// DOC servo2

// int servo2

// Set this variable to control servo 2 (must call init_expbd_servos first)

// DOC servo3

// int servo3

// Set this variable to control servo 3 (must call init_expbd_servos first)

// DOC servo4

// int servo4

// Set this variable to control servo 4 (must call init_expbd_servos first)

// DOC servo5

// int servo5

// Set this variable to control servo 5 (must call init_expbd_servos first)

// DOC set_beeper_pitch

// set_beeper_pitch(float: frequency)

// Sets the beeper pitch (use with beeper_on and beeper_off)

// DOC set_digital_out

// int set_digital_out(int)

// Sets expansion board digital output to logic high

// DOC sin

// float sin(float: angle)

// Sine of angle (radians)

// DOC sleep

// sleep(float: duration in seconds)

// Waits for given duration

// DOC sonar

// int sonar()

// Fires sonar and reports approximate distance in

// millimeters.  32767 means no echo detected.

// DOC sqrt

// float sqrt(float)

// Square root

// DOC start_button

// int start_button()

// Returns 1 if start button pressed, 0 if not

// DOC start_press

// start_press()

// Waits for start button to be pressed

// DOC start_process

// int start_process(function call)

// Starts a new process

// DOC stop_button

// int stop_button()

// Returns 1 if stop button pressed, 0 if not

// DOC stop_press

// stop_press()

// Waits for stop button to be pressed

// DOC tan

// float tan(float: angle)

// Tangent of angle (radians)

// DOC test_digital_out

// int test_digital_out(int: channel)

// Returns the last commanded state of the expansion board digital output

// DOC tone

// tone(float: frequency, float: duration in seconds)

// Makes a tone

// DOC track_orange

// track_orange()

// Sets track_ globals vars with camera trained to orange

// DOC track_blue

// track_blue()

// Sets track_ globals vars with camera trained to blue

// DOC trackRaw

// trackRaw (int: rmin, rmax, gmin, gmax, bmin, bmax)

// CMUcam tracks color defined by parameters

// DOC track_size

// int track_size

// Is # of pixels in image of color being tracked

// DOC track_x

// int track_x

// Is X coordinate of centroid of tracked color (-40,40)

// DOC track_y

// int track_y

// Is X coordinate of centroid of tracked color (-71,71)

// DOC track_area

// int track_area

// Size of bounding box of color being tracked

// DOC track_confidence

// int track_confidence

// Confidence of color being tracked